American University of Sharjah

College of Engineering

Computer Science & Engineering Department

Introduction to Computer Systems Lab COE 251L

Instructor: Ms. Donthi Sankalpa

Lab #10: Interrupts of 8086

# Objective

- To introduce the students to the concept of interrupts.

- Use pre-defined interrupt routines to read from and write to the console.

# Introduction

In this lab, you will be introduced to the concept of interrupts. An interrupt is a hardware-initiated subroutine call.  This gives the hardware access to software without any interaction until the hardware needs software to perform some function.

Interrupts

| Interrupts | Extra Registers | Description | Example |
| --- | --- | --- | --- |
| INT 21h / AH=9 | DX = Offset of msg data label | Output a string at DS:DX, String must be terminated by ‘$’ | .model small .stack 64 .data DATA_ASC DB "Hello World","$" newline db 13, 10, "$" .code main proc far mov ax, @data mov ds, ax MOV AH,09 ; option 09 to display string of data MOV DX,OFFSET DATA_ASC ;DX offset address of data INT 21H ;invoke the interrupt mov ah, 4ch int 21h end main |
| INT 21h / AH=2 | DL = Character to write (ASCII value) | Outputting a single character to the monitor | .model small .stack 64 .data .code main proc far mov ax, @data mov ds, ax MOV AH,02 ; option 02 displays one character MOV DL,"Y" ; DL holds the character to be displayed INT 21H ; invoke the interrupt. mov ah, 4ch int 21h end main |
| INT 21h / AH=1 | Character is stored in AL | Inputting a single character, with echo | .model small .stack 64 .data .code main proc far mov ax, @data mov ds, ax MOV AH, 01 INT 21H mov ah, 4ch int 21h end main |
| INT 10h / AH = 09h | AL = character to display. BH = page number. BL = attribute. CX = number of times to write character. | write character and attribute at cursor position Values for attribute: | .model small .stack 64 .data .code main proc far mov ax, @data mov ds, ax MOV AH, 09 MOV AL, 'S' MOV BH, 0 MOV BL, 1 ;Prints in blue MOV CX, 1 INT 10H mov ah, 4ch int 21h end main |

# Lab Work (1-2):

- Write an assembly program that first asks the user the size of the design (it is limited from 1 to 9) and then print the following half pyramid pattern. Hint: To convert the input from ascii to actual value you must subtract the value with ‘0’.  Use NEWLINE db 13, 10, "$" in the datasegment for newline.

Code:

.model small

.stack 64

.data

newline db 13, 10, "$"

promptmsg db "Enter the size of the pyramid (1-9): $"

invalidmsg db "Invalid size.", 13, 10, "$"

star db '*', "$"

.code

main proc

mov ax, @data

mov ds, ax

inputsize:

mov ah, 09h

lea dx, promptmsg

int 21h

mov ah, 01h

int 21h

sub al, '0'

cmp al, 1

jb invalidinput

cmp al, 9

ja invalidinput

mov cl, al

mov ah, 09h

lea dx, newline

int 21h

mov ch, 0

pyramidrow:

inc ch

mov bl, ch

printstars:

mov ah, 09h

lea dx, star

int 21h

dec bl

jnz printstars

mov ah, 09h

lea dx, newline

int 21h

cmp ch, cl

jb pyramidrow

mov ah, 4ch

int 21h

invalidinput:

mov ah, 09h

lea dx, newline

int 21h

mov ah, 09h

lea dx, invalidmsg

int 21h

jmp inputsize

main endp

end main

Screenshot:

- Write a program that asks the user to enter two 2 digit numbers and prints out wether num1 is greater than num2, num1 is lesser than num2 or that the two numbers are equal. You may assume the user only enter correct values. Make sure to submit 3 samples of outputs.

Code:

.model small

.stack 64

.data

NEWLINE db 13, 10, "$"

MSG1 db "Enter first 2-digit number: $"

MSG2 db "Enter second 2-digit number: $"

GREATER db "Input1 is greater than Input2.$", 13, 10, "$"

LESSER db "Input1 is lesser than Input2.$", 13, 10, "$"

EQUAL db "The inputs are equal.$", 13, 10, "$"

.code

main proc

mov ax, @data

mov ds, ax

mov ah, 09h

lea dx, MSG1

int 21h

mov ah, 01h

int 21h

sub al, '0'

mov bl, al

mov ah, 01h

int 21h

sub al, '0'

add bl, al

shl bl, 4

lea dx, NEWLINE

mov ah, 09h

int 21h

mov ah, 09h

lea dx, MSG2

int 21h

mov ah, 01h

int 21h

sub al, '0'

mov bh, al

mov ah, 01h

int 21h

sub al, '0'

add bh, al

shl bh, 4

lea dx, NEWLINE

mov ah, 09h

int 21h

cmp bl, bh

jg display_lesser

jl display_greater

display_equal:

lea dx, EQUAL

mov ah, 09h

int 21h

jmp end_program

display_lesser:

lea dx, LESSER

mov ah, 09h

int 21h

jmp end_program

display_greater:

lea dx, GREATER

mov ah, 09h

int 21h

end_program:

mov ah, 4Ch

int 21h

main endp

end main

Screenshot:

EXTRA QUESTION 3:

Code:

.model small

.stack 64

.data

promptmsg db "Enter your full name: $"

outputmsg db "Converted name: $"

newline db 13, 10, "$"

nameInput db 50 DUP('$')

.code

main proc

mov ax, @data

mov ds, ax

mov ah, 09h

lea dx, promptmsg

int 21h

mov bx, 0

readloop:

mov ah, 01h

int 21h

cmp al, 13

je convertcase

mov nameInput[bx], al

inc bx

jmp readloop

convertcase:

mov ah, 09h

lea dx, newline

int 21h

mov ah, 09h

lea dx, outputmsg

int 21h

mov cx, bx

mov si, 0

convertloop:

mov al, nameInput[si]

cmp al, 'a'

jb checkupper

cmp al, 'z'

ja checkupper

sub al, 32

jmp displaychar

checkupper:

cmp al, 'A'

jb storechar

cmp al, 'Z'

ja storechar

add al, 32

displaychar:

mov dl, al

mov ah, 02h

int 21h

storechar:

inc si

loop convertloop

mov ah, 09h

lea dx, newline

int 21h

mov ah, 4ch

int 21h

main endp

end main

Screenshot: